Project Guidelines — Student B

Learned Trajectory Policy for Real-Time Fairness-Aware UAV Control

# 1. Overview and Goal

Your project builds on the existing UAV-to-ground communications simulator (uav_extended_simulation.py), specifically Section 7, which models a dynamic scenario: a UAV serves several moving vehicle-users, each with its own data buffer and priority. The current strategy is a simple heuristic — at every time step the UAV moves toward the weighted centroid of the vehicles, with weights (buffer × priority) / distance. It is fast, but suboptimal.

We already know from Section 6 of the same project that the greedy approach does not deliver max-min fairness — it systematically leaves the "difficult" users (far away, in building shadows) with low rates. The theoretically correct solution would be to solve a max-min placement optimization at every time step. But that takes seconds of compute per decision — completely incompatible with real-time UAV control, where decisions are needed every 100 ms.

Your goal: train a policy network that learns to imitate a slow offline max-min planner, producing decisions in real time (forward pass < 5 ms) with quality close to the optimal planner. The technical approach is behavioral cloning — the simplest form of imitation learning, which is essentially supervised learning, avoiding the complexity of reinforcement learning. The main technical challenge is that the number of vehicles changes dynamically, so the network must be permutation-invariant — produce the same output regardless of the order in which vehicles are fed in.

# 2. Research Question

Primary question: Can a small permutation-invariant policy network, trained on the decisions of a slow offline max-min UAV planner, recover most of the expert's fairness quality while running in real time, and generalize to unseen vehicle counts and unseen motion patterns?

Secondary questions:

- How much better than the Section 7 greedy planner does the learned policy perform?

- Which architecture is enough — DeepSets or is self-attention required?

- How many expert trajectories are needed for a good policy? (data efficiency)

- Does it generalize to vehicle counts unseen during training?

- What distribution-shift problems appear? (the classic "compounding errors" of behavioral cloning)

# 3. Background — Imitation Learning

Imitation learning is a family of algorithms that learn policies by mimicking an expert. Unlike reinforcement learning, you do not need a reward function — you just have (state, expert_action) pairs and do supervised learning. The simplest form is behavioral cloning (BC).

## 3.1 Why Behavioral Cloning and Not RL

- Stability: RL is notoriously unstable — different seeds produce different results. BC is essentially supervised regression — if it has converged, it has converged.

- Speed: RL runs for hours or days. BC training finishes in minutes.

- Beginner-friendly: You do not need the lived experience required to tune PPO/SAC.

- No reward design: The expert defines "what is good."

## 3.2 The Drawback of BC: Distribution Shift

BC has a well-known problem: the agent is trained on states the expert visits. When deployed, small errors lead it to states never seen during training → larger errors. This is called "compounding errors" (Ross & Bagnell, 2010).

Mitigations you will try:

- Data augmentation with noise on the expert states (state perturbation)

- DAgger — a simple online correction that queries the expert when the policy enters new states (Stretch goal, only if you have time)

- Training over diverse scenarios so the state space is well covered

# 4. Methodology

## 4.1 The Expert

The expert you want to imitate is a max-min look-ahead planner. At every time step t, it solves the following problem:

Given:
  - Current positions/velocities of all vehicles
  - Buffers, priorities
  - Vehicle position prediction for the next H steps (constant velocity)

Find:
  - Sequence of H UAV positions (x_t, x_{t+1}, ..., x_{t+H})

Maximizing:
  min over vehicles of  sum_{tau=0..H} rate(uav_x_tau, vehicle_x_tau)

Subject to:
  ||x_{t+1} - x_t||  <=  V_max * dt   (UAV speed)
  z_min <= z_t <= z_max

In practice: use SLSQP from scipy.optimize with min-rate as the objective (smooth approximation via log-sum-exp so it is differentiable). Look-ahead horizon H = 5 steps is a good starting point. This takes 2–5 seconds per decision — slow, but acceptable for offline data generation.

## 4.2 Dataset Construction

Generate expert trajectories as follows:

- Random initial configuration: 5–10 vehicles with random positions, velocities, buffers, priorities

- UAV starts at the center of the area, at 100 m altitude

- Run a 60-second episode (60 steps × 1 second)

- At each step the expert plans and returns the first action (receding horizon)

- Store (state_t, expert_action_t) pairs

Dataset size target:

- 3000 episodes × 60 steps = 180,000 (state, action) pairs

- Approx. 25 hours of compute (0.5 s/decision, parallel over 8 cores)

## 4.3 State and Action Spaces

The state must be structured so that a permutation-invariant network can process it.

### State

Two parts:

- Per-vehicle features: tensor [N_vehicles, F] where F = (x, y, vx, vy, buffer_size, priority, distance_to_uav, predicted_rate). N_vehicles is variable.

- Global features: tensor [G] with current UAV position (x, y, z) and velocity (vx, vy)

### Action

2D UAV velocity for the next step: (vx, vy) ∈ [-V_max, V_max]². The z coordinate is fixed in this baseline. (Stretch goal: 3D with vz.)

## 4.4 Architecture

### 4.4.1 Baseline: DeepSets

The simplest permutation-invariant architecture. Each vehicle goes through the same φ-network, results are aggregated (mean, max, or sum), and finally combined with the global features:

class DeepSetsPolicy(nn.Module):
    def __init__(self, vehicle_dim=8, global_dim=5,
                 embed_dim=64, hidden=128):
        super().__init__()
        self.phi = nn.Sequential(
            nn.Linear(vehicle_dim, embed_dim), nn.ReLU(),
            nn.Linear(embed_dim, embed_dim), nn.ReLU(),
        )
        self.rho = nn.Sequential(
            nn.Linear(embed_dim + global_dim, hidden), nn.ReLU(),
            nn.Linear(hidden, hidden), nn.ReLU(),
            nn.Linear(hidden, 2),       # output: (vx, vy)
        )

    def forward(self, vehicles, global_feats, mask=None):
        # vehicles: [B, N, vehicle_dim], mask: [B, N] (1 = valid)
        embeddings = self.phi(vehicles)                   # [B, N, embed_dim]
        if mask is not None:
            embeddings = embeddings * mask.unsqueeze(-1)
            pooled = embeddings.sum(dim=1) / mask.sum(dim=1, keepdim=True).clamp(min=1)
        else:
            pooled = embeddings.mean(dim=1)
        combined = torch.cat([pooled, global_feats], dim=-1)
        return torch.tanh(self.rho(combined)) * V_MAX     # bounded action

About 30k parameters. Very simple, easy to debug.

### 4.4.2 Stretch Goal: Self-Attention

DeepSets takes a mean — it loses information about which vehicle is "critical". With self-attention, each vehicle can have different influence. Try this ONLY if DeepSets is performing acceptably and you have time (Steps 13–14):

class AttentionPolicy(nn.Module):
    def __init__(self, vehicle_dim=8, global_dim=5,
                 d_model=64, nhead=4, num_layers=2):
        super().__init__()
        self.vehicle_embed = nn.Linear(vehicle_dim, d_model)
        encoder_layer = nn.TransformerEncoderLayer(
            d_model=d_model, nhead=nhead,
            dim_feedforward=128, batch_first=True
        )
        self.encoder = nn.TransformerEncoder(encoder_layer, num_layers=num_layers)
        self.head = nn.Sequential(
            nn.Linear(d_model + global_dim, 128), nn.ReLU(),
            nn.Linear(128, 2),
        )

    def forward(self, vehicles, global_feats, mask=None):
        x = self.vehicle_embed(vehicles)
        x = self.encoder(x, src_key_padding_mask=~mask if mask is not None else None)
        # Mean pool over valid tokens
        if mask is not None:
            pooled = (x * mask.unsqueeze(-1)).sum(dim=1) / mask.sum(dim=1, keepdim=True).clamp(min=1)
        else:
            pooled = x.mean(dim=1)
        combined = torch.cat([pooled, global_feats], dim=-1)
        return torch.tanh(self.head(combined)) * V_MAX

## 4.5 Training

- Loss: MSE on the action — simple and clean

- Optimizer: AdamW, lr=3e-4, weight_decay=1e-4

- Batch size: 256 episodes (variable N → padding + masking)

- Epochs: 50–100 with early stopping

- Variable N handling: pad to N_max=10 and use a boolean mask so padded entries are ignored during pooling/attention

- Data augmentation: rotate/flip whole scenes, jitter vehicle positions with small Gaussian noise

## 4.6 Baselines

| Baseline | Description | What it shows |
| --- | --- | --- |
| Static center | UAV stays still at the centroid of initial vehicles | Floor — absolute worst case |
| Greedy centroid | The existing algorithm in Section 7 | The main baseline to beat |
| Random walk | Random motion within a bounding box | Shows the agent actually "learns" |
| MPC re-planning | The expert run live (slow) | Upper bound (quality) |
| Nearest-neighbor | Memorize training; at inference, copy the action of the nearest training state | Is memorization enough? |

## 4.7 Evaluation Metrics

- Min user rate: the primary fairness metric (max-min objective)

- Jain's fairness index: complementary fairness metric

- Sum-rate: to make sure you do not sacrifice too much throughput

- Inference time: wall-clock per decision — your killer feature

- Imitation gap: (expert_score - policy_score) / (expert_score - greedy_score) — fraction of the gap closed

# 5. Workflow — 16 Steps

The work is organized into 16 sequential steps grouped into four phases. Each step has a concrete deliverable. Move to the next step only when the current deliverable exists.

## Phase I — Expert and Data

### Step 1: Setup

- Clone the existing repo, clean up dependencies

- Meet with Student A to coordinate shared code (data loader, channel model)

- Read: Ross & Bagnell 2010 (DAgger), Zaheer et al. 2017 (DeepSets), Vaswani et al. 2017 (Attention)

- Deliverable: reading log, GPU access (Colab Pro or local), PyTorch "hello world"

### Step 2: Shared Infrastructure (with Student A)

- EUBUCCO data loader: build generate_town_eubucco together

- Fix Section 7: use the full channel model (Al-Hourani + shadowing + fading), not bare FSPL as today

- Set up a shared config system (Hydra or OmegaConf)

- Deliverable: clean shared code, working on 3 cities

### Step 3: Expert Implementation

- Implement the max-min look-ahead planner

- Compare with the greedy planner on 5 toy scenarios — demonstrate it is genuinely better

- Profile: how long per decision? (expected 2–5 s)

- Deliverable: MaxMinExpert class with documented superiority over greedy

### Step 4: Data Generation Pipeline

- Generate 200 episodes as a smoke test

- Parallelize (joblib with 8 workers)

- Storage format: HDF5 or Parquet (large dataset, efficient I/O matters)

- Validation: visualize 5 expert trajectories — do they look reasonable?

- Deliverable: pipeline that can generate 3000 episodes in < 24 h

## Phase II — Modeling

### Step 5: Full Data Generation

- Generate 3000 episodes (180k samples)

- Train/val/test split: 80/10/10 (split by episode, not by step)

- EDA: action distribution, vehicle counts, scenarios

- Deliverable: final HDF5 dataset, EDA notebook

### Step 6: Simple Baselines

- Static center, greedy, random — all on the same test split

- Metrics: min rate, Jain's index, inference time

- Deliverable: baseline results table

### Step 7: DeepSets Training

- Implement DeepSetsPolicy

- Training loop with logging (wandb preferred)

- Hyperparameter sweep: lr, embed_dim, hidden, pooling type (mean/max/sum)

- Deliverable: trained DeepSets, comparison against baselines

### Step 8: Evaluation

- Run the trained policy on 100 test episodes

- Imitation gap calculation, fairness analysis

- Failure modes: when does the policy break? (visualize trajectories)

- Deliverable: initial results, clear evidence that DeepSets beats greedy

## Phase III — Generalization and Integration

### Step 9: Generalization Studies

- Train with N=5–7 vehicles, test with N=3 and N=10 (out-of-distribution)

- Data scaling: train with 500, 1000, 2000, 3000 episodes

- Deliverable: generalization curves

### Step 10: Attention Model (if on track)

- Implement AttentionPolicy

- Compare DeepSets vs Attention

- If Attention is meaningfully better → kept for the paper; if not → keep DeepSets, mention attention as ablation

- Deliverable: architecture comparison table

### Step 11: Robustness and Noise

- How does the policy behave under noisy state observations?

- Under delayed actions? (10–50 ms latency simulation)

- Stretch: a simple DAgger pass (1–2 iterations only)

- Deliverable: robustness analysis

### Step 12: Integration with Student A

- Replace "true" channel computation in the state with Student A's predictions

- Joint experiment: policy + simulator vs policy + NN surrogate

- Show the policy remains effective when fed predicted channel info

- Deliverable: joint pipeline end-to-end, joint comparison plot

## Phase IV — Writing and Polish

### Step 13: Paper Outline and Figures

- Six-page outline (workshop format)

- Killer figures: trajectory snapshots/animation comparing the 4 policies side-by-side

- Imitation-gap plot, inference-time bar chart

### Step 14: First Draft

- Full first version of the paper

- Supervisor comments

### Step 15: Revisions and Joint Paper Merge

- Revisions, figure polish

- Work with Student A on the combined paper sections

### Step 16: Final Submission

- Camera-ready, anonymization if needed

- Repo cleanup with reproducibility instructions

- Animated GIF of the trajectory comparison for the demo

# 6. Connection with Student A's Work

Important: Your work is not standalone. At the end, the two projects merge into a single story — "Learning end-to-end UAV networking: from channel surrogates to learned policies." This means you need to coordinate with Student A from the very first step.

## 6.1 What You Share

- Shared repo: one GitHub repo with clear branches (main, student-a/*, student-b/*) and shared modules under /common/

- Shared data loader: generate_town_eubucco is shared. Build it together in Steps 1–2.

- Shared channel model: fixing Section 7 to use the full channel model is co-owned. It matters to you directly because your expert trajectories need an accurate channel.

- Shared config system: same interface for reproducibility

## 6.2 How the Pieces Connect at the End

Student A's work is a neural surrogate that predicts UAV-to-user link capacity fast and accurately. In your state space, one of the per-vehicle features is "predicted rate from the UAV" — which today is computed by the slow physical simulator. In the joint pipeline:

- Your policy consumes rate predictions coming from Student A's surrogate

- A fully real-time pipeline is achieved: state collection → channel prediction (1 ms) → policy decision (5 ms) → action

- Without the surrogate, state collection takes 100+ ms per decision — the pipeline is not real-time

Specifically, in Step 12 you will jointly run the following experiments:

- Policy + original simulator (slow baseline, ground-truth quality)

- Policy + Student A's NN surrogate (fast, the realistic deployment)

- Compare: imitation gap, inference time, end-to-end pipeline latency

If the above shows that the full ML pipeline (surrogate + policy) achieves fairness comparable to the original simulator + expert but runs in real time, the two of you have a strong joint story. That is the mission.

## 6.3 Sync Cadence

Meetings:

- Weekly meeting with the supervisor (30 minutes)

- Additional Student A + Student B sync every other week (shared-code review)

- Slack channel for ad-hoc questions

Critical coordination milestones:

- End of Step 2: the data loader is ready and validated by both of you

- End of Step 2: Section 7 channel model fixed — this matters to you directly

- End of Step 8: Student A's surrogate is ready for your policy to consume

- End of Step 12: joint experiment runs end-to-end

# 7. Final Deliverables

- 1. Workshop-style paper draft (6 pages, IEEE template or similar)

- 2. Clean repo with README, requirements.txt, and one-command reproduction

- 3. Trained model checkpoints (DeepSets + Attention if reached)

- 4. Animated GIF/video comparing the 4 policy trajectories

- 5. 15-minute talk to the lab

# 8. Risks and Mitigation

| Risk | Likelihood | Mitigation |
| --- | --- | --- |
| The expert (SLSQP) does not converge reliably | Medium | Multiple random initializations, time budget, fallback to greedy on failure |
| Data generation is too slow | Medium | Fewer episodes (1500 instead of 3000), parallelization |
| Compounding errors at deployment | High | Classic BC weakness. State-noise augmentation, possibly DAgger |
| Does not generalize to different N | Medium | Train with diverse N. If still fails → curriculum learning |
| Student A falls behind | Medium | Your work stands alone if needed; the joint paper is a bonus |

# 9. Reading List

Read all of these in the first two steps of work. Deep dive is not required — general understanding is.

### Must-read

- Ross, Gordon, Bagnell (2011). "A reduction of imitation learning and structured prediction to no-regret online learning." AISTATS. (DAgger)

- Zaheer et al. (2017). "Deep Sets." NeurIPS.

- Vaswani et al. (2017). "Attention is all you need." NeurIPS.

### UAV / wireless context

- Al-Hourani et al. (2014). "Optimal LAP altitude for maximum coverage."

- Mozaffari et al. (2019). "A tutorial on UAVs for wireless networks."

- Bayerlein et al. (2020). "UAV path planning for wireless data harvesting." (RL for UAV — for context)

### Practical imitation learning

- Pomerleau (1989). "ALVINN." (original BC paper — short, classic)

- Codevilla et al. (2018). "End-to-end driving via conditional imitation learning."

- Florence et al. (2022). "Implicit Behavioral Cloning." CoRL. (if you want the current state of the art)

# 10. Closing Remarks

This work has every ingredient for a workshop or student-track paper:

- A clear research question with a measurable claim (imitation gap + 1000× speedup)

- Strong baselines (greedy, static, random, expert) — the claim is not empty

- Permutation-invariant architecture (technical depth, valued in reviews)

- Generalization studies across different N

- Connection to a larger story (via Student A)

Two warnings:

- Do not switch to RL. It is tempting to think "I will do PPO/SAC instead of BC". For a beginner with limited time, this is a trap. BC works; RL might never work.

- Do not attempt multi-UAV. One UAV serves many vehicles. Multi-agent is a separate research area — it can be a follow-up paper.

The secret to making this publishable is to NOT try to do everything. Do exactly what this document says, and do it well. With the time available, "well" is everything.

Good luck — let's do excellent work.